Construction Tech 1

Cold Metal Processes part 2

Mr. Patterson

- Breaking- bending or forming a flat shape into a 3 dimensional object.

  - 2 main forms of breaking metal

    - Hand breaking-using specially designed hand tool ( hand seamer) to bend sheet metal

    - Machine Breaking

      - Press break

      - Box Break

      - Pan Break

Fastening Metal

- Drilling

- Taping

- Riveting

- Bolting

Drilling Procedure

  - Layout the center of the hole to be drilled

  - Mark the center with two intersecting lines

  - Center punch the intersecting lines

  - Select the correct drill size from a drill index

  - Put the drill into the drill chuck and tighten with the chuck key.

- Clamp material to the table

- Place a drop of cutting fluid on the center punch mark

- Turn on drill and begin to drill hole

- Be sure to put even pressure on the drill

Cutting Threads

- There are 2 ways to cut threads:

  - External Threads (bolt)

    - Die-cutting threads on the outside of a piece of rod

  - Internal Threads (nut)

    - Tap-cutting threads on the inside of a hole

Tapping Procedure

- Layout hole location using “Drilling” procedure

- Select required tap size and threads per inch (TPI)

- Consult Tap Drill chart for hole size

Threads inside and out

- 3 classifications of threads

  - National Fine (UNF)

  - National Course (UNC)

  - National Pipe Thread (NPT)

- Thread Diameter

  - Diameter of the thread measured directly over the threads

- Thread Pitch- # of threads per inch

Threading Tools

- Tap

- Tap handle

- Die

- Die Stock

- Cutting Fluid