CLF351

- -   AGRICULTURAL CORE CURRICULUM  - -

(CLF300)  Core Area:  PLANT SCIENCE

(CLF350)  Unit Title:  IRRIGATION

____________________________________________________________________________

(CLF351)   Topic:  IRRIGATION              Time      Taught in Year(s)

SYSTEMS                2 hours           1

____________________________________________________________________________

Topic Objectives:  Upon completion of this lesson the student will

be able to:

Learning Outcome #:

(D-1)  -  Name four types of irrigation systems.

(D-4)  -  Demonstrate two methods of irrigation in a plant

production situation.

Special Materials and Equipment: A collection of sprinkler heads

and drip irrigation emitters.

References: Janick, Schery, Woods, and Ruttan.(1981).  Plant

Science (third edition).San Francisco CA:  McGraw-

Hill Book Co. Pp. 393-408

Hartman, Kofraneck, Rubatzk, and Plocker.  (1988)

Plant Science (second edition).  Englewood Cliffs,

NJ:  Prentice-Hall.  Pp 200-204.

California Department of Water Resources

(educational resources library)

Metropolitan Water District-Southern California

(extensive and easily obtained educational

resources).

Evaluation: Quiz by instructor

TOPIC PRESENTATION:  IRRIGATION SYSTEMS

A.  Because a very large percentage of the earth's land mass receives less

than 20 inches of rain per year, the need for irrigation is very

obvious.

B.  Farmers have irrigated crops for over 4,000 years.

1.  Records show that crops were irrigated along the Euphrates, Ganges,

Nile and Tigris rivers as early as 2600 B.C.

2.  Great civilizations develop when the population no longer needs to

lead a nomadic life-style.  A portion of the population produces all

of the food so other members can perform other specialized

activities ranging from making tools to establishing governments.

Irrigation, no doubt, contributed to the founding of these great

civilizations.

351.1

C.  Even though irrigation has been practiced for so many years, modern

methods were started as little as 200 years ago.

D.  Irrigation Systems:

1.  Water requirements vary depending on the:

a.  crop,

b.  climate,

c.  season,

d.  soil conditions, and

e.  method of application.

2.  Selection of the right watering system can save labor and water,

and assure increased crop yields.

3.  There are specific reasons for selecting one of the basic four

methods of irrigation, namely:

a.  border or flood,

b.  furrow,

c.  sprinkler, and

d.  drip.

4.  Each of these have characteristics which makes them useable in a

specific situation.

a.  Border:  (flood) irrigation is used where the land is level.

1)  This method is often used for orchards and vineyards and for

hay, pasture and cereal grains.

2)  The land must be leveled and graded.

3)  A uniform slope of 0.1 to 0.4 ft/100 ft. (0.1 - 0.4 % slope)

is used for most crops.

4)  Water enters through a head ditch or biplane and is released

into the individual checks (areas bounded by levees running

downslope) by siphons, gates, or valves.

b.  Furrow irrigation is one of the most widely used methods for

crops.

1)  Water runs down the furrows between plant rows.

2)  Water moves to all parts of the soil by capillary action or

gravity.

351.2

3)  This system is efficient in water utilization, but is

expensive because of high labor costs.  Costs include:

a)  forming the furrows;

b)  forming and maintaining the irrigations ditches; and

c)  hiring help to maintain the ditches and irrigation cycle

using gates, siphons, etc.

4)  An advantage of furrow irrigation is uniform crop maturity,

possible because water is more uniformly applied than in

flood irrigation.

c.  Sprinkler irrigation is often used when flood or furrow systems

are impractical.

1)  Leveling is not as critical for sprinkler irrigation.

2)  More uniform wetting of the surface area is possible.

a)  Seed germination is more uniform.

b)  Salts can be washed away.

c)  Less total water is used, which can increase efficiency.

3)  Sprinklers can also be used for frost control.

4)  The slower rate of application of this system reduces

erosion, run off and soil compaction.

5)  In general, initial installation costs and power costs are

higher than for flood or furrow irrigation systems.

d.  Drip irrigation is the latest innovation in irrigation systems.

1)  The soil is wetted by small amounts of water dripped

(trickled) slowly through devices called emitters.

2)  Emitters are located at intervals along a lateral plastic

tube which is usually laid on the soil surface.

3)  The lateral lines are connected to a main line that receives

water from a source.

4)  Water is usually applied very slowly at the rate of 1 to 2

gallons per hour per emitter under low pressure.

5)  The main advantage of this system is that it requires less

water than the other systems.

6)  The delivery system is light weight, being easily moved by

one person; but is normally a solid set and not moved often.

351.3

7)  Fertilizers and some pesticides can be added to the water

before delivery to crops.

8)  It can be used on steeper slopes than other systems.

9)  Disadvantages to the drip system are:

a)  high cost of installation;

b)  a need for increased maintenance;

c)  root distribution of perennial crops changes to adjust

to the new system and therefore there is a temporary

loss of production; and

d)  salts tend to accumulate on the soil surface and along

the wetted boundaries.

__________________________________________________________

ACTIVITY:

1.  Take field trips in your local area to farms using

each of these methods of irrigation - flood, furrow,

sprinkler and drip.

2.  When you're on your way to a judging contest, state

finals, etc., stop and discuss types of irrigation

systems visible from the highway.  (When you cross the

California Aqueduct, point it out to the students

and point out its value and the organization of the

delivery system.)

3.  A collection of sprinkler heads and drip emitters can

be shared with and demonstrated for the class.

4.  Demonstrate four methods of irrigation in a plant

production situation.

__________________________________________________________

351.4