CLF354

- -   AGRICULTURAL CORE CURRICULUM  - -

(CLF300)  Core Area:  PLANT SCIENCE

(CLF350)   Unit Title:  IRRIGATION

____________________________________________________________________________

(CLF354)   Topic:  WATER MEASUREMENT          Time      Taught in Year(s)

AND SOIL CAPACITY         1 hour             2

____________________________________________________________________________

Topic Objectives:  Upon completion of this lesson the student will

be able to:

Learning Outcome #:

(D-2)         -   Explain (and demonstrate) the concept of water

holding (or field) capacity of different soils.

(D-3,4; G-4)  -  Discuss acre inch, acre foot, water penetration

and water holding capacity.

Special Materials and Equipment: Sample problems for student

practice; 4 potted plants.

Evaluation: Quiz by instructor

TOPIC PRESENTATION:  WATER MEASUREMENTS AND SOIL CAPACITY

A.  The cost of irrigation water varies widely throughout the U.S and the

world.

1.  Individual crops require a varying amount of water to reach

marketable size, stage or maturity.

2.  Perennial crops will require 36 inches or more of useable

(available) water.

3.  Our concern with this natural resource, water, requires study and

understanding.

4.  Lets take a look at how water is measured and apply this to

agricultural or garden uses.

B.  When we hear a rainfall report it is given in inches of water

"precipitated."  What does this mean?

1.  One inch of water in the form of rainfall over an area means that

the entire area received enough rain to cover it with a depth of one

inch of water.

2.  If this were measured over a given area of one acre, we would have

received one "acre inch of water."

354.1

3.  Each time it rains during the season the additional precipitation

(in inches) is added to the total.

4.  As an example, lets say that during our rainy season we received 12

inches of rain.

5.  This would mean that we have received enough water to cover each

acre with one foot of water (one acre foot).

6.  Typically water is measured in ACRE FEET.

a.  In irrigation districts, the landowner is charged a given amount

for each acre foot of water s/he uses.  Example:

1)  Grower A uses 1.75 acre feet of water.  The cost is $50.00

per acre foot.

2)  The total cost per acre paid by the landowner is $87.50

C.  In irrigated areas growers supplement the natural precipitation (rain or

snow) with irrigation water.

1.  If a crop requires 32 inches of water to reach maturity and it has

rained 12 inches, how much water would the grower have to add by

irrigation?

a.  32 inches minus 12 inches = 20 inches of water needs to be

added.

1)  This would be the case for 100% efficiency of water use.

2)  However, that is seldom the case, the best is 85-90% and

common is 65-75%

3)  Therefore, water needs for the crop would be more than 20

inches of water and would be closer to 22 to 30 inches of

water.

__________________________________________________________

ACTIVITY:

Why isn't plant use of water 100% efficient.  Set up the

following demonstration:

Identify factors that affect evaporation and transpiration

rates.  Set up a simple experiment using 4 similar size

potted plants.  Evenly water all four plants, wait for the

excess to drain and weight them.  Place two plants

outside for 24 hours, keep the other two in the classroom.

Reweigh all plants the next day and discuss the reasons

for the differences in weights.

__________________________________________________________

2.  What would be the per acre cost of the irrigation water at $50.00

per acre foot?

354.2

a.  20 inches = 1.66 feet,

b.  so, 1.66 ft times $50.00 per acre foot = $83.00 per acre.

D.  What is the depth of WATER PENETRATION for one inch of water?

1.  This will vary with the type of soil.

2.  Typical ranges of penetration for each type of soil are:

a.  10 to 12 inches penetration in sand,

b.  6 to 10 inches penetration in silt, and

c.  4 to 6 inches penetration in clay.

3.  Factors that affect water penetration include:

a.  soil texture,

b.  organic matter content of the soil,

c.  presence of impermeable layers,

d.  amount of soil compaction, and

e.  the amount of surface crusting.

E.  The ability of a soil to retain (hold) water is called the water-holding

capacity.

__________________________________________________________

ACTIVITY:

Demonstrate field (or water holding) capacity with a pot

of loamy soil and water.  Fill a container with water until

flooded, and allow it to drain until no free water flows.

Have students handle the drained soil to develop an

understanding of what field capacity is.  Contrast these

observations with a container of sand.

__________________________________________________________

1.  The water holding capacity of soils will vary, especially with soil

texture.

2.  Typical water holding capacities (water per foot of soil) by soil

textures are:

a.  .5 to .75 inches per foot of sandy soil,

b.  .75 to 1.25 inches per foot of loam soils, and

c.  1.25 to 2.0 inches per foot of clay soils.

354.3

__________________________________________________________

ACTIVITY:

1.  Have students work typical water measurement problems

similar to those shown in text of this topic.

2.  Have students determine cost of irrigation water using

examples similar to those used in topic outline.

__________________________________________________________

354.4